examination paper

| SUBJECT: CERTIFICATE IN ROCK MECHANICS: PAPER 2 SUBJECT CODE: COMRME ### EXAMINATION DATE: OCTOBER 2010 TIME: 9:00 – 12:00 | EXAMINER: K BRENTLEY MODERATOR: J LUCAS TOTAL MARKS: [100] PASS MARK: 60% |
| --- | --- |

NUMBER OF PAGES: 12

SPECIAL REQUIREMENTS: - Answer ALL FIVE questions - References other than those provided are not permitted. - Hand-held electronic calculators may be used. - Write your examination number on the outside cover of each book used and on any graph paper or other loose sheets handed in. NB: your name must not appear on any answer book or loose sheets. - Write in ink on the RIGHT HAND SIDE of the paper only (only the right hand pages will be marked). - Show all calculations on which your answers are based. - Illustrate your answers by sketches of diagrams wherever possible. - In answering these questions, full advantage should be taken wherever necessary of your practical experience as well as of the data given. - Answers must be given to an accuracy that is typical of practical conditions.

ANSWERS  -  QUESTION 1

Multiple Questions

| 1.1 | A change in deformation with time at a constant stress is known as: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | a) |  | Strain rate | b) |  | Creep (Jager & Ryder p.107) |  |  |  |
|  | c) |  | Time dependent failure | d) |  | Anisotropic behaviour |  | (1) |  |
| 1.2 | Calculate the maximum bord width using L = 2 + 0.15 BESR where: L = 2.6m (bolt length) ESR = 2 (risk of excavation), and B = bord width |  |  |  |  |  |  |  |  |
|  | a) |  | 6m | b) |  | 5.0m |  |  |  |
|  | c) |  | 10m | d) |  | 8m |  | (2) |  |
|  | L = 2 + 0.15 B ESR B = ESR L-2 0.15 |  |  |  |  |  |  |  |  |
| 1.3 | What monitoring instrument is used to measure vertical alignment of shaft barrels as well as surface subsidence over areas of shallow mining? |  |  |  |  |  |  |  |  |
|  | a) |  | Extensometer | b) |  | Accelerometer |  |  |  |
|  | c) |  | Tilt meter (Ryder & Jager p.462) | d) |  | Vernier gauge |  | (1) |  |
| 1.4 | What is the effective pillar width for a pillar measuring 4m x 3m? |  |  |  |  |  |  |  |  |
|  | a) |  | 3m | b) |  | 3.43m |  |  |  |
|  | c) |  | 4m | d) |  | 0.86m |  | (1) |  |
| 1.5 | Which of the following mining methods would result in the most strain energy being stored in the near field? |  |  |  |  |  |  |  |  |
|  | a) |  | VCR Stoping | b) |  | Cut and Fill |  |  |  |
|  | c) |  | Sub-level Caving | d) |  | Room and Pillar (Brady & Brown p.301) |  | (1) |  |
| 1.6 | True or False: The seismic Schmidt Number can be easily derived from recorded wave forms. |  |  |  |  |  |  |  |  |
|  | a) |  | True | b) |  | False (It is model dependent) |  | (1) |  |
| 1.7 | The optimum backfill slurry relative density normally lies between: |  |  |  |  |  |  |  |  |
|  | a) |  | 1.2 – 1.4 | b) |  | 1.4 – 1.6 |  |  |  |
|  | c) |  | 1.6 – 1.8 | d) |  | 1.8 – 2.0 |  | (1) |  |
| 1.8 | When considering literature with respect to shaft infrastructure in hard rock mines, at what induced vertical millistrains would you expect concrete lining to be compromised in compression? |  |  |  |  |  |  |  |  |
|  | a) |  | 0 – 0.2 | b) |  | 0.21 – 0.4 |  |  |  |
|  | c) |  | 0.4 – 0.69 | d) |  | Above 0.7 |  | (1) |  |
| 1.9 | In a small future open cast mine, the projected stripping ratio is calculated at ________?. Where the: - Total volume of the planned pit is 1 200 000 tons - Total volume of the planned ore is 600 000 tons |  |  |  |  |  |  |  |  |
|  | a) | 1 |  |  | b) |  | 2 |  |  |
|  | c) | 0.5 |  |  | d) |  | 4 | (1) |  |
| 1.10 | In the Mohr circle failure envelop for intact rock which is presented below, which of the representations are correct for points A, B and C? |  |  |  |  |  |  |  |  |
|  | a) |  |  |  |  |  |  |  |  |
|  | b) |  |  |  |  |  |  |  |  |
|  | c) |  |  |  |  |  |  |  |  |
|  | d) |  |  |  |  |  |  | (1) |  |
| 1.11 | Determine the modulus of rigidity of the material if the Young’s modulus is 100 GPa and the Poisson’s ratio is 0.25 (E = G (2(1 + v )) G = E 2 (1+v) |  |  |  |  |  |  |  |  |
|  | a) | 60 |  |  | b) |  | 40 |  |  |
|  | c) | 250 |  |  | d) |  | 100 | (2) |  |
| 1.12 | When testing the strength of rock samples and drastically increasing the rate of loading, what will be the result? |  |  |  |  |  |  |  |  |
|  | a) |  | Strength increase | b) |  | Strength decrease |  |  |  |
|  | c) |  | No effect | d) |  | Discing |  | (1) |  |
| 1.13 | In seismological terms, the “initial rupture point within a seismic source” is called the: |  |  |  |  |  |  |  |  |
|  | a) |  | Focus | b) |  | Epicentre |  |  |  |
|  | c) |  | Pole | d) |  | Hypocentre |  | (1) |  |
| 1.14 | In seismic source calculation the D = Mo (6.28 GR2) is known as: |  |  |  |  |  |  |  |  |
|  | a) |  | McGarr’s model | b) |  | Brune’s model |  |  |  |
|  | c) |  | Durrheim’s model | d) |  | Spottiswoode model |  | (1) |  |
| 1.15 | Determine the wave length (m) of a large seismic event where the P- velocity is 6km/s and the dominal frequency of a Magnitude 2 event is 20 Hz with C = L |  |  |  |  |  |  |  |  |
|  | a) |  | 300 | b) |  | 0.3 |  |  |  |
|  | c) |  | 30 | d) |  | 3000 |  | (1) |  |
| 1.16 | In the Mohr’s mineral hardness classification scale, what are the missing two stones? Talc Gypsum Calcite Fluorite Apatite Feldspar _______ ________ Corundum Diamond |  |  |  |  |  |  |  |  |
|  | a) |  | Quartz, Hematite | b) |  | Quartz, Topaz |  |  |  |
|  | c) |  | Graphite, Topaz | d) |  | Hematite, Graphite |  | (1) |  |
| 1.17 | When studying metamorphic rocks, it is evident that Marble is formed from: |  |  |  |  |  |  |  |  |
|  | a) |  | Sandstone | b) |  | Limestone |  |  |  |
|  | c) |  | Shale | d) |  | Mudstone |  | (1) |  |
| 1.18 | As rocks are quite weak in tension, it is important to understand their tensile strength. As a rough estimate (rule of thumb), the tensile strength will be ______ % of the compressive strength. |  |  |  |  |  |  |  |  |
|  | a) |  | 5 % | b) |  | 10 % |  |  |  |
|  | c) |  | 15 % | d) |  | 20 % |  | (1) |  |
|  |  |  |  |  |  |  |  |  | [20] |

QUESTION 2

Pillars

| 2.1 | Name and define the points indicated on the stress-strain curve of a pillar. | (6) | [6] |
| --- | --- | --- | --- |

| 2.1 | A - Yield Stress or Yield Point Indicates the on-set of localized inelastic behaviour or failure of some material in the pillar. | (2) |  |
| --- | --- | --- | --- |
|  | B - Strain Hardening The pillar can sustain further permanent damage without loosing its load carrying capacity (deformation becomes irrecoverable). | (2) |  |
|  | C - Residual Strength The minimum strength of the pillar which is generally reached after post-peak deformation. | (2) | [6] |

| 2.2 | Bavians Mine extracts ore from a tabular deposit situated some 1 000m below surface. The support system consists of crush pillars and timber elongates. |  |  |
| --- | --- | --- | --- |
|  | Given: Stope width = 1,4m a = 0,5 b = -0,75 Density = 3 000kg/m3 UCS = 200 MPa Insitu “cube strength” = 0,33 UCS Gravity = 10m/s2 |  |  |
|  | - Calculate the percentage extraction that can be expected for the pillar layout. | (2) |  |

|  | i) % Extraction = Area Supported – Pillar Area Area Supported = 363m 2 – 24m 2 363m 2 = 93.4 % x 100 x 100 | (2) |  |
| --- | --- | --- | --- |

|  | - Calculate the support resistance and height of hangingwall supported where the residual pillar strength is one tenth of the pillar strength. | (10) |  |
| --- | --- | --- | --- |

|  | ii) Area supported by pillar = 363m2 Area of pillar = 24m2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Pillar Strength = kwa hb = 66 x 4.360,5 1.40,75 = 107 MPa | Effective width = 4 x Area Circumference = 4 x 24m2 22m2 = 4,36m2 | (2) |  |
|  | Residual Pillar Strength = Pillar strength x 0,1 = 107 MPa . 0.1 = 10.7 MPa (11MPa) |  | (2) |  |
|  | Available Pillar Load = Residual Pillar Strength x Pillar area = 10.7 MPa x 24m2 = 256.8 MN (256 800 kN) |  | (2) |  |
|  | Support Resistance = Pillar Load Area supported by pillar = 256.8 mN 363m2 = 0.708 mN/m2 (708 kN/m2) |  | (2) |  |
|  | Height of Hangingwall = Support Resistance Density x Gravity = 0.708 mN/m2 3000 kg/m2 x 10 m/s2 = 23.6m |  | (2) | [12] |

|  | - Give two functions of a crush pillar. | (2) | [14] |
| --- | --- | --- | --- |

|  | iii) - Support the tensile zone between regional pillars - Provide support of the dead weight up to any well defined partings in the hangingwall. | (2) | [2] |
| --- | --- | --- | --- |

|  |  |  | [20] |
| --- | --- | --- | --- |

QUESTION 3

Numerical Modelling

| 3.1 | Numerical modelling is one of the most powerful tools available to the rock engineer to assist in finding solutions to design problems. |  |  |
| --- | --- | --- | --- |
|  | Discuss the process you would follow in setting up a numerical modelling exercise. | (10) |  |

| 3.1 | Jager and Ryder, pg 402 |  |  |
| --- | --- | --- | --- |
|  | i) Decide on what the purpose of the modelling exercise is to be. (1) You could write down the questions you need (1) answered and then make sure you answer them when the modeling work is completed. |  |  |
|  | ii) Identify and justify assumptions (1) e.g. 2D or 3D (1) model, elastic (is it adequate?). |  |  |
|  | iii) Select appropriate (1) modelling technique or techniques i.e. computer or (1) calculations using basic mechanic principles. |  |  |
|  | iv) Consider the trade off between accuracy and speed of solution (1). |  |  |
|  | v) Model must include important aspects of the physical situation e.g. non-linear material behavior, dynamics, etc. (1) |  |  |
|  | vi) Boundary conditions and size of region modeled must be carefully selected. (1) |  |  |
|  | vii) Check results against simplified solutions or practical solutions to ensure output stresses and displacements are in the correct range. (1) | (10) |  |

| 3.2 | If you were to apply the term “Occam’s Razor” to your modelling, what would it mean? | (3) |  |
| --- | --- | --- | --- |

| 3.2 | Jager and Ryder, pg 330 |  |  |
| --- | --- | --- | --- |
|  | Eliminate all unnecessary information relating to the problem that is being analysed. Applied mainly in the early stages of the model. Cuts down on excessive processing time and complex inputs and outputs. | (3) |  |

| 3.3 | Write the correct letter next to the modelling package which best represents the program used. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Besol UDEC Flac Map3D Minap |  |  |  |  |
|  | A - Distinct Element B - Finite Difference C - Displacement Discontinuity D - Fictitious Stress and Displacement Discontinuity E - Finite Element |  |  | (5) |  |

| 3.3 | Jager and Ryder, pg 342 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Besol UDEC Flac Map3D Minap | C A B C D |  | (5) |  |

| 3.4 | Name two types of continuum materials that can be modelled. | (2) |  |
| --- | --- | --- | --- |

| 3.4 | Gases Liquids (any 2) Solid | (3) |  |
| --- | --- | --- | --- |

|  |  |  | [20] |
| --- | --- | --- | --- |

QUESTION 4

Rockmass Characterisation

| 4.1 | Sketch and/or describe 5 (five) essential features on how rock specimens are prepared for compressive strength tests. | (5) |  |
| --- | --- | --- | --- |

| 4.1 | Ryder & Jager, pg 57-58 |  |  |
| --- | --- | --- | --- |
|  | i) Width-to-height ratio of 0.5 or 0.33 (1) (coal specimens could be in a cube form or cylinder form with a width : height ratio of 1). |  |  |
|  | ii) Ends of the specimen should be prepared to a high degree of parallelity (1) (0,0005mm/mm length of specimen). |  |  |
|  | iii) Minimum specimen diameter of 54mm (1) (at least 10 times the maximum grain size). |  |  |
|  | iv) Should be cored such that the (1) axial loading is applied normal to the bedding. |  |  |
|  | v) Specimens to be loaded at a constant (1) rate of 0.5mm/min (stress rate 0,5 to 1 MPa/s). |  |  |
|  | vi) Capping materials or other end (1) surface materials should not be used. |  |  |
|  | vii) Axial load, axial and radial strains / deformations should be recorded throughout the test. (1) |  |  |
|  | viii) Specimens should not be stored longer than 30 days. (1) |  |  |
|  | ix) No discontinuities or flaws. (1) | (5) |  |

| 4.2 | Describe what is meant by a “soft” testing machine and a “stiff” testing machine used to do compressive tests on rock core in a laboratory. | (4) |  |
| --- | --- | --- | --- |

| 4.2 | Soft testing machine - Elastic energy stored in the frame of the testing machine causes the rock specimen to fail violently. | (2) |  |
| --- | --- | --- | --- |
|  | Stiff testing machine - The machine is electrically and hydraulically controlled to prevent the elastic energy build-up and the specimen fails in a controlled manner. (Ryder and Jager, pg 80) | (2) |  |

| 4.3 | What is the main benefit of conducting compressive tests in a “stiff” testing machine? | (2) |  |
| --- | --- | --- | --- |

| 4.3 | The complete load / deformation curve can be established i.e. load shedding of specimen and residual strength. (Ryder and Jager, pg 80) | (2) |  |
| --- | --- | --- | --- |

| 4.4 | Sketch and describe the “Brazilian tensile test” method. | (5) |  |
| --- | --- | --- | --- |

| 4.4 |  |  |  |
| --- | --- | --- | --- |
|  | - Horizontally aligned cylindrical specimen. - Radius preferably >25mm. - Length of specimen = Radius (ℓ = R) - Compressed between two flat (or slightly curved) cylindrical plates. - Indirect “tensile strength” results: 3 = -P/R= -t | (5) |  |

| 4.5 | An alternative to the Brazilian tensile test is to directly pull a core specimen apart or use a “Dogbone” shaped specimen. List 4 (four) disadvantages of this testing method. | (4) |  |
| --- | --- | --- | --- |

| 4.5 | (Ryder & Jager, Page 65) |  |  |
| --- | --- | --- | --- |
|  | - Stress concentrations at points of application (tensile pulling forces). - Specimen can have no flaws (joints, etc.) - Specimen requires tedious machining to get “dogbone” shape. - Very expensive method. | (4) |  |

|  |  |  | [20] |
| --- | --- | --- | --- |

QUESTION 5

Drilling and Blasting

| 5.1 | Discuss the mode of action of explosives with reference to the shock wave and the gas pressure effects. | (4) |  |
| --- | --- | --- | --- |

| 5.1 | Immediately around the borehole, the shock wave causes compressive failure and crushing of the rock. Around the crushed zone, radial tensile fractures will develop due to the tensile tangential stress fracturing the rock. The fractures will propagate preferentially in the direction of the maximum principal stress. The gases will also try to escape down these radial fractures and will cause them to propagate further. | (4) |  |
| --- | --- | --- | --- |
|  | Pulverised - compression Radial fracture - tangential tensile Radial - gas pressure propagation |  |  |

| 5.2 | Drilling and blasting can have a profound effect on strata control in stopes. List and discuss the main factors and indicate their significance as far as strata control is concerned. | (4) |  |
| --- | --- | --- | --- |

| 5.2 | The most common problem is blast fractures extending into the hangingwall. These cause blocky ground and may lead to falls of ground. |  |  |
| --- | --- | --- | --- |
|  | Main factors leading to blast damage are: |  |  |
|  | - Length of hole. The shorter the hole the smaller the burden of misdirected holes and the smaller the probability of holes being drilled skew. (Length stoping width) |  |  |
|  | - Hole direction - large burden more damage. - drilling into hangingwall. |  |  |
|  | - Over-charging - more gas more damage. |  |  |
|  | - Burden - no breaking more damage. |  |  |
|  | e) Timing out of sequence burden increase more damage. |  |  |
|  | - Type of explosives gas vs. shock | (4) |  |

| 5.3 | Smoothwalling techniques are used for developing tunnels when blast damage to the wall rock needs to be minimized. One of the requirements for successful smoothwall blasting is the de-coupling of the explosive charges. |  |  |
| --- | --- | --- | --- |
|  | - Briefly explain how the de-coupling of explosives assist in decreasing blast damage. | (2) |  |

|  | - The air gap absorbs large amounts of energy from the blast and reduces shattering. The gas pressure effect is reduced which reduces the spread of fractures. | (2) |  |
| --- | --- | --- | --- |

|  | - If the overall coupling is defined as the ratio of the volume occupied by the explosive to the volume of the blast hole, calculate the spacing between explosive cartridges to achieve an overall coupling of 20 percent using the following data: | (8) |  |
| --- | --- | --- | --- |
|  | ii) |  |  |

|  |  | (8) |  |
| --- | --- | --- | --- |

| 5.4 | What is the composition of Anfex / Anfo explosives and list the estimated percentages. | (2) |  |
| --- | --- | --- | --- |

| 5.4 | 94% Ammonium nitrate 6% Diesel | (2) |  |
| --- | --- | --- | --- |

|  |  |  | [20] |
| --- | --- | --- | --- |